|  | iMaster NCE-Super V100R020C10 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Northbound REST API Guide |  |  |  |
|  | Issue | 04 |  |  |
|  | Date | 2021-10-15 |  |  |
|  | HUAWEI TECHNOLOGIES CO., LTD. |  |  |  |

Copyright © Huawei Technologies Co., Ltd. 2021. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written consent of Huawei Technologies Co., Ltd. Trademarks and Permissions and other Huawei trademarks are trademarks of Huawei Technologies Co., Ltd. All other trademarks and trade names mentioned in this document are the property of their respective holders. Notice The purchased products, services and features are stipulated by the contract made between Huawei and the customer. All or part of the products, services and features described in this document may not be within the purchase scope or the usage scope. Unless otherwise specified in the contract, all statements, information, and recommendations in this document are provided "AS IS" without warranties, guarantees or representations of any kind, either express or implied. The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute a warranty of any kind, express or implied.

| Huawei Technologies Co., Ltd. |  |
| --- | --- |
| Address: | Huawei Industrial Base Bantian, Longgang Shenzhen 518129 People's Republic of China |
| Website: | https://www.huawei.com |
| Email: | support@huawei.com |

Preface

Purpose

This document describes in detail the docking instructions for the northbound REST interface as well as the REST API reference for each domain in the iMaster NCE-Super V100R020C10 release.

Intended Audience

This guide is intended for the following engineers:

Technical support engineers.

System engineers.

Symbol Conventions

The symbols that may be found in this document are defined as follows.

| Symbol | Description |
| --- | --- |
|  | Indicates a hazard with a high level of risk which, if not avoided, will result in death or serious injury. |
|  | Indicates a hazard with a medium level of risk which, if not avoided, could result in death or serious injury. |
|  | Indicates a hazard with a low level of risk which, if not avoided, could result in minor or moderate injury. |
|  | Indicates a potentially hazardous situation which, if not avoided, could result in equipment damage, data loss, performance deterioration, or unanticipated results. NOTICE is used to address practices not related to personal injury. |
|  | Supplements the important information in the main text. NOTE is used to address information not related to personal injury, equipment damage, and environment deterioration. |

GUI Conventions

The GUI conventions that may be found in this document are defined as follows.

| Convention | Description |
| --- | --- |
| Boldface | Buttons, menus, parameters, tabs, windows, and dialog titles are in boldface. For example, click OK. |
| > | Multi-level menus are in boldface and separated by the ">" signs. For example, choose File > Create > Folder. |

Command Conventions

The command conventions that may be found in this document are defined as follows.

| Convention | Description |
| --- | --- |
| Boldface | The keywords of a command line are in boldface. |
| Italic | Command arguments are in italic. |
| [ ] | Items (keywords or arguments) in square brackets [ ] are optional. |
| { x ¦ y ¦ ... } | Optional items are grouped in braces and separated by vertical bars. One is selected. |
| [ x ¦ y ¦ ... ] | Optional items are grouped in square brackets and separated by vertical bars. One or none is selected. |
| { x ¦ y ¦ ... } * | Optional items are grouped in braces and separated by vertical bars. A minimum of one or a maximum of all can be selected. |
| [ x ¦ y ¦ ... ] * | Optional items are grouped in square brackets and separated by vertical bars. A maximum of all or none can be selected. |

Change History

| Issue | Date | Description |
| --- | --- | --- |
| 04 | 2021-10-15 | This issue is the fourth official release. |
| 03 | 2021-07-30 | This issue is the third official releas |
| 02 | 2021-05-15 | This issue is the second official release. |
| 01 | 2021-05-15 | This issue is the first official release. |

Contents

Preface	ii

1 API Description	6

1.	Common APIs	6

1.1.1 Alarm Management	6

1.1.1.1 Report Alarm Severity Change Notifications	6

1.1.1.2 Query Static Alarm Information	11

1.1.1.3 Query Alarm Statistics	17

1.1.1.4 Query Current and Historical Alarms	20

1.1.1.5 Acknowledge or Unacknowledge Alarms	33

1.1.1.6 Query Masked Alarms	36

1.1.1.7 Query Alarms based on Service Information	39

1.1.1.8 Update Alarm Information	43

API Description

  - Common APIs

Alarm Management

The northbound notification service of NCE provides alarm query and operation APIs.

Report Alarm Severity Change Notifications

Typical Scenarios

An OSS needs to receive alarm severity change notifications. By default, NCE does not report such notifications.

Functions

Report alarm severity change notifications.

Constraints

None.

HTTP Method

POST

URI

/restconf/v1/data/ietf-alarms:alarms/control

Request Parameters

body parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| control | Yes | REFERENCE | For details, see Table 1-2. |  | Parameters determining whether to report alarm severity change notifications. | - |

control parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| x733-mapping | No | ARRAY_REFERENCE | For details, see Table 1-3. |  | X.733 parameter defining the alarm types supported by the system. (The default X.733 parameter can be modified.) This parameter is reserved. | - |
| alarm-shelving | No | REFERENCE | For details, see Table 1-5. |  | Alarm filtering rule. This parameter is reserved. | - |
| max-alarm-status-changes | No | string | - | 32 | Length of the alarm status change list. This parameter is reserved. | - |
| notify-status-changes | Yes | boolean | - | false | Whether to report alarm severity changes. | false |

x733-mapping parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| alarm-type-qualifier-match | No | string | - | - | Type of the alarm to be mapped. | - |
| x733-alarm-definition-parameters | No | REFERENCE | For details, see Table 1-4. |  | X.733 parameters defined in the alarm. | - |
| alarm-type-id | No | string | - | - | ID of the alarm to be mapped. | - |

x733-alarm-definition-parameters parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| probable-cause | No | int64 | - | - | Alarm cause. This parameter is reserved. For details about possible causes, see probable-cause in alarm-parameters. | - |
| event-type | No | string | other-alarm communications-alarm quality-of-service-alarm processing-error-alarm equipment-alarm environmental-alarm integrity-violation-alarm operational-violation-alarm physical-violation-alarm security-service-or-mechanism-violation-alarm time-domain-violation-alarm | - | Alarm type defined by ITU-T. | - |

alarm-shelving parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| shelf | No | ARRAY_REFERENCE | For details, see Table 1-6. |  | Alarm filtering rule. | - |

alarm-shelving-shelf parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| alarm-type-qualifier-match | No | string | - | - | Alarm filtering by type (regular expression). | - |
| alarm-type-id | No | string | - | - | Alarm filtering by type ID. (Correlative alarms related to the alarm ID are also returned.) | - |
| resource | No | array<string> | - | - | List of resources. | - |
| name | No | string | - | - | Name of an alarm filtering rule. | - |
| description | No | string | - | - | Description of the alarm filtering rule, which should describe the reason for configuring this filtering rule. | - |

Sample Request

HTTP

POST /restconf/v1/data/ietf-alarms:alarms/control 
Content-Type: application/json 
Accept: application/json 
Accept-Language: en-US 
X-Auth-Token: x-rwld6ng9vy2rg96llg85sbdisbmrfw1gdfamaoepoahcaq2nliams8s8epapiofx2r5druhi5g7wmmjzc5btftlduo7w88bybuqp7xmlup9htd5gpg4bmmrufvnt4aan 
{ 
  "notify-status-changes": "true" 
}

Response Parameters

Response status code 200: Successful operation

Response status code 400: Bad request

For details, see the response message body.

Sample Response

Response status code 200: Successful operation

HTTP/1.1 200 OK

Response status code 400: Bad request

HTTP/1.1 400 Bad Request 
Date: Mon,30 Dec 2019 10:00:00 GMT 
Server: example-server 
Content-Type: application/json;charset=UTF-8 
Content-Length: 250 
Connection: keep-alive

Query Static Alarm Information

Typical Scenarios

An OSS needs static alarm information for fault O&M.

Functions

Query static alarm information.

Constraints

Maximum concurrency of the API: 2.

HTTP Method

GET

URI

/restconf/v1/data/ietf-alarms:alarms/alarm-inventory

query parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| limit | No | int32 | [1-40000] | - | Number of records on each page. The value ranges from 1 to 40000. If this parameter is not set, the first 40000 alarms will be returned by default. | - |
| marker | No | string | 36 characters. The value must meet the following regular expression: [0-9a-fA-F]{8}-[0-9a-fA-F]{4}-[0-9a-fA-F]{4}-[0-9a-fA-F]{4}-[0-9a-fA-F]{12}. | - | Iteration sub-index. | - |

Request Parameters

N/A

Sample Request

HTTP

GET /restconf/v1/data/ietf-alarms:alarms/alarm-inventory 
Content-Type: application/json 
Accept: application/json 
Accept-Language: en-US 
X-Auth-Token: x-rwld6ng9vy2rg96llg85sbdisbmrfw1gdfamaoepoahcaq2nliams8s8epapiofx2r5druhi5g7wmmjzc5btftlduo7w88bybuqp7xmlup9htd5gpg4bmmrufvnt4aan

Response Parameters

Response status code 200: Successful operation

alarm-inventory parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| alarm-type | ARRAY_REFERENCE | For details, see Table 1-9. |  | Alarm type. | - |

alarm-inventory-alarm-type parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| severity-levels | array<string> | Each item has indeterminate minor warning major critical | - | List of alarm severities. | - |
| x733-alarm-definition-parameters | REFERENCE | For details, see Table 1-10. |  | X.733 parameters defined in the alarm. | - |
| resource | array<string> | - | - | List of types of resources that may report this alarm. | - |
| description | string | 0 to 65535 characters. | - | Alarm description (including possible causes and handling suggestions). | "Remote Alarm Indication" |
| alarm-type-qualifier | string | 0 to 256 characters. | - | Dynamically defined alarm type, which works when the alarm type cannot be statically defined by alarm-type-id. | "268374017-125" |
| has-clear | boolean | - | - | Whether the alarm can be automatically cleared. | false |
| alarm-type-id | string | other-alarm communications-alarm quality-of-service-alarm processing-error-alarm equipment-alarm environmental-alarm integrity-violation-alarm operational-violation-alarm physical-violation-alarm security-service-or-mechanism-violation-alarm time-domain-violation-alarm | - | Description of the alarm type (static). | "security-service-or-mechanism-violation-alarm" |
| alarm-definition-parameters | REFERENCE | For details, see Table 1-11. |  | Extended parameters in northbound static alarm information, compared with IETF. | - |

x733-alarm-definition-parameters parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| probable-cause | int64 | - | - | Alarm cause. This parameter is reserved. For details about possible causes, see probable-cause in alarm-parameters. | - |
| event-type | string | other-alarm communications-alarm quality-of-service-alarm processing-error-alarm equipment-alarm environmental-alarm integrity-violation-alarm operational-violation-alarm physical-violation-alarm security-service-or-mechanism-violation-alarm time-domain-violation-alarm | - | Alarm type defined by ITU-T. | - |

alarm-definition-parameters parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| native-probable-cause | string | - | - | Alarm name, which briefly describes the related fault symptom. | - |
| probable-cause | string | - | - | Possible causes of the alarm. | - |
| reason-id | int64 | - | - | Alarm cause ID. (One alarm may have multiple causes. This parameter is used to identify the specific cause of the alarm.) | - |
| repair-action | string | - | - | Alarm handling suggestions. | - |

Sample Response

Response status code 200: Successful operation

HTTP/1.1 200 OK 
Content-Type: application/json   
{ 
  "alarm-type": [{ 
    "severity-levels": [ 
      "warning" 
    ], 
    "x733-alarm-definition-parameters": { 
      "event-type": "environmental-alarm" 
    }, 
    "resource": [], 
    "description": "Loopback, terminal", 
    "alarm-type-qualifier": "268374024-43585", 
    "has-clear": true, 
    "alarm-type-id": "environmental-alarm", 
    "alarm-definition-parameters": { 
      "native-probable-cause": "LPBKTERM", 
      "probable-cause": "[WDM]\nTerminal Loopback is enabled.", 
      "reason-id": 43585 
    } 
  }] 
}

Query Alarm Statistics

Typical Scenarios

An OSS needs to obtain alarm statistics from NCE, including the total number of alarms, number of cleared alarms at each severity, and number of uncleared alarms.

Functions

Query alarm statistics.

Constraints

The number of concurrent interfaces is 3.

HTTP Method

GET

URI

/restconf/v1/data/ietf-alarms:alarms/summary

Request Parameters

N/A

Sample Request

HTTP

GET /restconf/v1/data/ietf-alarms:alarms/summary 
Content-Type: application/json 
Accept: application/json 
Accept-Language: en-US 
X-Auth-Token: x-rwld6ng9vy2rg96llg85sbdisbmrfw1gdfamaoepoahcaq2nliams8s8epapiofx2r5druhi5g7wmmjzc5btftlduo7w88bybuqp7xmlup9htd5gpg4bmmrufvnt4aan

Response Parameters

Response status code 200: Successful operation

summary parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| alarm-summary | ARRAY_REFERENCE | For details, see Table 1-13. |  | Statistics on all alarms on the system (excluding masked alarms). | - |
| shelves-active | string | - | - | Whether to enable alarm masking. This parameter is reserved. | - |

alarm-summary parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| not-cleared-closed | int32 | - | - | Number of uncleared but closed alarms at the specified severity. This parameter is reserved. | - |
| severity | string | indeterminate minor warning major critical | - | Alarm severity (corresponding to key in alarm-summary). | - |
| not-cleared-not-closed | int32 | - | 0 | Number of uncleared and unclosed alarms at the specified severity. | - |
| cleared-not-closed | int32 | - | 0 | Number of cleared but unclosed alarms at the specified severity. This parameter is reserved. | - |
| total | int32 | - | 0 | Number of alarms at the specified severity. | - |
| cleared | int32 | - | 0 | Number of cleared alarms at the specified severity. | - |
| cleared-closed | int32 | - | 0 | Number of cleared and closed alarms at the specified severity. This parameter is reserved. | - |

Sample Response

Response status code 200: Successful operation

HTTP/1.1 200 OK 
Content-Type: application/json   
{ 
  "alarm-summary": [ 
    { 
      "not-cleared-closed": 0, 
      "severity": "critical", 
      "not-cleared-not-closed": 67, 
      "total": 1192, 
      "cleared": 1125 
    }, 
    { 
      "not-cleared-closed": 0, 
      "severity": "major", 
      "not-cleared-not-closed": 218, 
      "total": 1238, 
      "cleared": 1020 
    }, 
    { 
      "not-cleared-closed": 0, 
      "severity": "minor", 
      "not-cleared-not-closed": 34, 
      "total": 182, 
      "cleared": 148 
    }, 
    { 
      "not-cleared-closed": 0, 
      "severity": "warning", 
      "not-cleared-not-closed": 32, 
      "total": 276, 
      "cleared": 244 
    } 
  ] 
}

Query Current and Historical Alarms

Typical Scenarios

An OSS needs to obtain cleared and uncleared alarms from NCE. If there are a large number of alarms, the query results are automatically paginated. By default, only the first 1000 alarms are returned, and the URL of the next page is attached to the header in the response. Considering the volume of cleared alarms, NCE queries the cleared alarms in the past three days and all uncleared alarms by default. To query earlier alarms, specify the start time. To narrow down the query scope, you are advised to specify the start time as required.

Functions

Query cleared and uncleared alarms.

Constraints

1. The interface supports a maximum of two concurrent calls.

2. In pagination mode, the filter criteria (is-cleared, alarm-type-qualifier, perceived-severity, resource, and alarm-serial-number) for each query must be the same as those for the first query. The filter criteria are verified only during the first query.

3. The number of query parameters delivered by the interface is restricted by the URL length. For details about the URL length, see "URL Description.".

HTTP Method

GET

URI

/restconf/v1/data/ietf-alarms:alarms/alarm-list

query parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| is-cleared | No | boolean | true false | - | Whether to query cleared alarms. If this parameter is not set, all alarms on NCE will be returned. | - |
| limit | No | int32 | [1-5000] | 1000 | Number of records on each page. The value ranges from 1 to 5000. If this parameter is not set, the first 1000 alarms will be returned by default. | - |
| marker | No | string | 36 characters. The value must meet the following regular expression: [0-9a-fA-F]{8}-[0-9a-fA-F]{4}-[0-9a-fA-F]{4}-[0-9a-fA-F]{4}-[0-9a-fA-F]{12}. | - | Iteration sub-index. | - |
| alarm-type-qualifier | No | array<string> | - | - | Alarm ID. | - |
| perceived-severity | No | array<string> | Each item has critical major minor warning | - | Alarm severity. | - |
| resource | No | array<string> | - | - | Alarm source, that is, resource UUID. If an NE UUID is delivered, alarms on the NE and all sub-objects of the NE will be returned. | - |
| start-time | No | string | - | - | Start time. It works in conjunction with end-time. The format is 2019-07-10T00:00:00Z. | - |
| end-time | No | string | - | - | End time. It works in conjunction with start-time. The format is 2019-07-13T00:00:00Z. | - |
| alarm-serial-number | No | array<string> | - | - | Alarm SN. | - |

Request Parameters

header parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| fileLocationURI | No | string | 1 to 255 characters. | - | SFTP path to which alarm files are exported. Specify a real path, for example, sftp://root:root@10.22.44.148:22/home/ftproot/alarm.json. | "sftp://root:root@10.22.44.148:22/home/ftproot/alarm.json" |

N/A

Sample Request

HTTP

GET /restconf/v1/data/ietf-alarms:alarms/alarm-list?limit=2000&is-cleared=false&start-time=2019-05-30T06:35:38Z&end-time=2019-05-30T14:35:38Z 
Content-Type: application/json 
Accept: application/json 
Accept-Language: en-US 
X-Auth-Token: x-rwld6ng9vy2rg96llg85sbdisbmrfw1gdfamaoepoahcaq2nliams8s8epapiofx2r5druhi5g7wmmjzc5btftlduo7w88bybuqp7xmlup9htd5gpg4bmmrufvnt4aan

Response Parameters

Response status code 200: Successful operation

alarm-list parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| alarm | ARRAY_REFERENCE | For details, see Table 1-17. |  | List of alarms. | - |
| last-changed | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Last modification time of the alarm list, in format of 2019-03-30T08:21:46.000Z. This parameter is reserved. | - |
| number-of-alarms | string | - | - | Number of alarms. This parameter is reserved. | - |

alarm parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| time-created | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Alarm generation time, that is, time when the alarm is generated for the first time or generated again after being cleared. The time format is 2019-03-30T08:21:46.000Z. If alarm status (is-cleared, perceived-severity, or alarm-text) changes, the time-created node will not be affected but the last-changed node will be updated. | "2019-03-30T08:21:46.000Z" |
| is-acked | boolean | - | - | Acknowledgment status of the alarm. | false |
| resource-alarm-parameters | REFERENCE | For details, see Table 1-18. |  | Alarm parameters in the resource view. | - |
| x733-alarm-parameters | REFERENCE | For details, see Table 1-20. |  | X.733 parameters of the alarm. | - |
| operator-state-change | ARRAY_REFERENCE | For details, see Table 1-21. |  | History of alarm operations performed by operators. | - |
| alarm-parameters | REFERENCE | For details, see Table 1-22. |  | Extended parameters in the northbound alarm model, compared with IETF. | - |
| common-alarm-parameters | REFERENCE | For details, see Table 1-23. |  | General alarm attributes. | - |

resource-alarm-parameters parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| perceived-severity | string | indeterminate minor warning major critical | - | Current alarm severity. | "major" |
| is-cleared | boolean | - | - | Clearance status. | false |
| status-change | ARRAY_REFERENCE | For details, see Table 1-19. |  | Alarm change history, including is-cleared, perceived-severity, and alarm-text changes. | - |
| last-changed | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Latest update time, in format of 2019-03-30T08:21:46.000Z. Change scenarios include is-cleared, perceived-severity, and alarm-text. For a fault alarm, enter its occurrence time. For a clear alarm, enter the clearance time. This parameter does not reflect the time of severity changes. | "2019-03-30T08:21:46.000Z" |

alarm-state-change-parameters parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| perceived-severity | string | indeterminate minor warning major critical | - | Alarm severity (changeable). | "critical" |
| alarm-text | string | 0 to 256 characters. | - | Alarm status change. This parameter takes effect only when the alarm experiences clearance, acknowledgment, or severity changes. | "Alarm cleared." |
| time | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Time when the alarm status changes, in the format of 2019-03-30T08:21:46.000Z. This parameter takes effect only when an event or alarm is reported, or an alarm experiences clearance, acknowledgment, or severity changes. | "2019-03-30T08:21:46.000Z" |

x733-alarm-parameters parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| probable-cause | int64 | - | - | Alarm cause. This parameter is reserved. For details about possible causes, see probable-cause in alarm-parameters. | - |
| event-type | string | other-alarm communications-alarm quality-of-service-alarm processing-error-alarm equipment-alarm environmental-alarm integrity-violation-alarm operational-violation-alarm physical-violation-alarm security-service-or-mechanism-violation-alarm time-domain-violation-alarm | - | Alarm type defined by ITU-T. | "equipment-alarm" |

operator-parameters parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| operator | string | 0 to 256 characters. | - | Alarm operator. | "nbitest" |
| text | string | - | - | Additional operation information. | - |
| state | string | none ack shelved unshelved closed | - | Alarm status from operators' perspective. The options are as follows: - none - ack - shelved - unshelved - closed | "ack" |
| time | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Operation time, in format of 2019-03-30T08:21:46.000Z. | "2019-03-30T08:21:46.000Z" |

alarm-parameters parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| repair-action | string | - | - | Alarm handling suggestions. | - |
| ne-name | string | 0 to 768 characters. | - | Name of the NE where the alarm is generated. If the alarm is reported from an NE, enter the NE name. If the alarm is reported from a system service, enter OSS. If the alarm is reported from the performance module, enter an NE name. | "NE(9-95)" |
| location-info | string | 0 to 2250 characters. | - | Alarm location information, which cannot be parsed and is in format of 8-N1EDQ41-5(SDH-5)-VC4:5-VC12:16[1-6-1]. | "0-subrack-108-U1PSU-OTHER" |
| native-probable-cause | string | 0 to 768 characters. | - | Alarm name, which briefly describes the related fault symptom. | "POWER_FAIL" |
| probable-cause | string | 0 to 65535 characters. | - | Possible causes of the alarm. | "(1)The standby power is off;(2)The power board is failed;(3)The power supply of the board is aged." |
| root-cause-identifier | boolean | - | - | Whether it is a root alarm. | false |
| ems-time | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Time when the alarm arrives at the EMS, in format of 2019-03-30T08:21:46.000Z. | "2019-03-30T08:21:46.000Z" |
| alarm-serial-number | string | 0 to 64 characters. | - | Alarm log SN. | "39621" |
| reason-id | int64 | - | - | Alarm cause ID. (One alarm may have multiple causes. This parameter is used to identify the specific cause of the alarm.) | 125 |
| tenant-id | string | 0 to 1536 characters. | - | Tenant ID, which is involved only in the tenant scenario. | "00000000-0000-0000-0000-000000000000" |
| tenant | string | 0 to 1536 characters. | - | Tenant name, which is involved only in the tenant scenario. | "DefaultOrganization" |
| alarm-text | string | 0 to 65535 characters. | - | Alarm description, which should describe the fault symptom and provide handling suggestions. | "Power failure" |
| other-info | string | 0 to 750 characters. | - | Alarm additional information | "OrderNumber=ITSM-9FLOIBMX2PV1KU8C, Src=*.*.*.*, Dst=*.*.*.*, Index=31, Alarm Parameter II(hex) 0x00, ss=rrrrr" |
| ip-address | string | - | - | IP address of the alarm source | "127.0.0.1" |

common-alarm-parameters parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| alt-resource | array<string> | Each item has 0 to 256 characters. | - | List of other identification modes for the resource object where the alarm is generated. | [] |
| resource | string | The value must meet the following regular expression: \\w{8}(-\\w{4}){3}-\\w{12}¦OSS. | - | Resource object of the alarm. | "ad3028ba-c7d4-11ea-ad00-286ed489e617" |
| resource-url | string | 0 to 4096 characters. | - | URL of the object associated with the alarm. | "/restconf/v2/data/huawei-nce-resource-inventory:network-elements/network-element/ad3028ba-c7d4-11ea-ad00-286ed489e617" |
| related-alarm | ARRAY_REFERENCE | For details, see Table 1-24. |  | Associated alarm. | - |
| alarm-type-qualifier | string | 0 to 256 characters. | - | Alarm type UUID. | "100000-299" |
| impacted-resource | array<string> | Each item has 0 to 1536 characters. | - | List of the resource objects that may be affected by the alarm. | - |
| root-cause-resource | array<string> | Each item has 0 to 256 characters. | - | List of resource objects for the root alarm. | - |
| alarm-type-id | string | other-alarm communications-alarm quality-of-service-alarm processing-error-alarm equipment-alarm environmental-alarm integrity-violation-alarm operational-violation-alarm physical-violation-alarm security-service-or-mechanism-violation-alarm time-domain-violation-alarm | - | Description of the alarm type. | "security-service-or-mechanism-violation-alarm" |
| layer | string | 0 to 256 characters. | - | Service layer. | "LR_Not_Applicable" |
| md-name | string | 0 to 256 characters. | - | Name of the system that sends the alarm. | "Huawei/NCE" |
| product-type | string | 0 to 192 characters. | - | NE type. | "HUAWEI OSN902" |

related-alarm parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| root-alarm-serial-number | string | - | - | SN of the root alarm. | - |
| alarm-type-qualifier | string | - | - | Type of the associated alarm. | - |
| resource | string | - | - | Resource identifier of the associated alarm. | - |
| alarm-type-id | string | - | - | ID of the associated alarm. | - |

Response status code 400: Bad request

For details, see the response message body.

Sample Response

Response status code 200: Successful operation

HTTP/1.1 200 OK 
Content-Type: application/json   
{ 
  "alarm": [{ 
    "time-created": "2019-06-10T08:46:15.000Z", 
    "is-acked": false, 
    "resource-alarm-parameters": { 
      "perceived-severity": "major", 
      "is-cleared": false, 
      "status-change": [], 
      "last-changed": "2019-06-10T08:46:15.000Z" 
    }, 
    "x733-alarm-parameters": { 
      "event-type": "equipment-alarm" 
    }, 
    "operator-state-change": [], 
    "alarm-parameters": { 
      "ne-name": "7-241", 
      "location-info": "0-subrack-108-U1PSU-OTHER", 
      "native-probable-cause": "POWER_FAIL", 
      "probable-cause": "(1)The standby power is off;\n(2)The power board is failed;\n(3)The power supply of the board is aged.", 
      "root-cause-identifier": false, 
      "ems-time": "2019-06-10T13:59:06.853Z", 
      "alarm-serial-number": "39621", 
      "reason-id": 125, 
      "tenant-id": "", 
      "tenant": "", 
      "alarm-text": "Power failure" 
    }, 
    "common-alarm-parameters": { 
      "alt-resource": [], 
      "resource": "91a94f76-8b6f-11e9-853d-b77f2b1e7375", 
      "resource-url": "/restconf/v2/data/huawei-nce-resource-inventory:network-elements/network-element/91a94f76-8b6f-11e9-853d-b77f2b1e7375", 
      "related-alarm": [], 
      "alarm-type-qualifier": "268374017-125", 
      "impacted-resource": [], 
      "root-cause-resource": [], 
      "alarm-type-id": "equipment-alarm" 
    } 
  }] 
}

Response status code 400: Bad request

HTTP/1.1 400 Bad Request 
Date: Mon,30 Dec 2019 10:00:00 GMT 
Server: example-server 
Content-Type: application/json;charset=UTF-8 
Content-Length: 250 
Connection: keep-alive

Acknowledge or Unacknowledge Alarms

Typical Scenarios

An OSS needs to acknowledge or unacknowledge alarms on NCE.

Functions

Acknowledge or unacknowledge alarms by a combination of resource, alarm-type-id, or alarm-type-qualifier or by alarm SN. If an SN is specified, NCE acknowledges or unacknowledges the corresponding alarm. If no SN is specified, NCE acknowledges or unacknowledges alarms based on the filter criteria in the URL.

Constraints

None.

HTTP Method

POST

URI

/restconf/v1/operations/ietf-alarms:alarms/alarm-list/alarm/{alarm-key}/set-operator-state

path parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| alarm-key | Yes | string | The value must meet the following regular expression: (\\w{8}(-\\w{4}){3}-\\w{12}-¦OSS-¦-)(([a-z]{1,}-){1,}alarm)-([0-9]{1,}-[0-9]{1,}). | - | Identifier consisting of resource, alarm-type-id, and alarm-type-qualifier, where resource indicates the alarm source UUID, alarm-type-id indicates the alarm type description, and alarm-type-qualifier indicates the alarm type ID. | "2327c028-51f9-11e9-87dd-790924e8efc5-communications-alarm-268374026-13523" |

Request Parameters

body parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| set-operator-state | Yes | REFERENCE | For details, see Table 1-27. |  | Alarm O&M status. | - |

set-operator-state-input parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| input | Yes | REFERENCE | For details, see Table 1-28. |  | Alarm O&M status. | - |

set-operator-state-input-body parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| text | Yes | string | 0 to 256 characters. | - | Additional operation information. | "nbitest" |
| state | Yes | string | none ack | - | Alarm O&M status. The options are as follows: - none - ack | "ack" |
| alarm-serial-number | No | string | - | - | Alarm SN. | - |

Sample Request

HTTP

POST /restconf/v1/operations/ietf-alarms:alarms/alarm-list/alarm/2327c028-51f9-11e9-87dd-790924e8efc5-communications-alarm-268374026-13523/set-operator-state 
Content-Type: application/json 
Accept: application/json 
Accept-Language: en-US 
X-Auth-Token: x-rwld6ng9vy2rg96llg85sbdisbmrfw1gdfamaoepoahcaq2nliams8s8epapiofx2r5druhi5g7wmmjzc5btftlduo7w88bybuqp7xmlup9htd5gpg4bmmrufvnt4aan 
{ 
  "input": { 
    "text": "nbitest", 
    "state": "ack", 
    "alarm-serial-number": "66997" 
  } 
}

Response Parameters

Response status code 200: Successful operation

Response status code 400: Bad request

For details, see the response message body.

Sample Response

Response status code 200: Successful operation

HTTP/1.1 200 OK

Response status code 400: Bad request

HTTP/1.1 400 Bad Request 
Date: Mon,30 Dec 2019 10:00:00 GMT 
Server: example-server 
Content-Type: application/json;charset=UTF-8 
Content-Length: 250 
Connection: keep-alive

Query Masked Alarms

Typical Scenarios

An OSS needs to query the alarms that it has not received from NCE.

Functions

Query masked alarms, including the alarms masked by NCE and the alarms masked by NBIs. For the alarms masked by NCE, is-cleared is false. For the alarms masked by NBIs, is-cleared is consistent with the alarm clearance status on NCE.

Constraints

1. This interface supports only a maximum of two concurrent calls.

2. When a northbound filtering rule has been configured and enabled in the Alarm Monitor app (Alarm Settings > Northbound Filtering Rules), the "Query Masked Alarms" interface needs to traverse all active and historical alarms before sorting out the alarms that do not need to be reported to the OSS. This move will increase the response time of the interface.

3. When neither start-time nor end-time is set, masked alarms in the past 3 days are returned by default.

HTTP Method

GET

URI

/restconf/v2/data/ietf-alarms:alarms/shelved-alarms

query parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| limit | No | int32 | [1-5000] | 1000 | Number of records on each page. If this parameter is not set, the first 1000 alarms will be returned by default. | - |
| marker | No | string | 36 characters. The value must meet the following regular expression: [0-9a-fA-F]{8}-[0-9a-fA-F]{4}-[0-9a-fA-F]{4}-[0-9a-fA-F]{4}-[0-9a-fA-F]{12}. | - | Iteration sub-index. | - |
| start-time | No | string | - | - | Start time. It works in conjunction with end-time. The format is 2019-07-10T00:00:00Z. | - |
| end-time | No | string | - | - | End time. It works in conjunction with start-time. The format is 2019-07-13T00:00:00Z. | - |

Request Parameters

N/A

Sample Request

HTTP

GET /restconf/v2/data/ietf-alarms:alarms/shelved-alarms 
Content-Type: application/json 
Accept: application/json 
Accept-Language: en-US 
X-Auth-Token: x-rwld6ng9vy2rg96llg85sbdisbmrfw1gdfamaoepoahcaq2nliams8s8epapiofx2r5druhi5g7wmmjzc5btftlduo7w88bybuqp7xmlup9htd5gpg4bmmrufvnt4aan

Response Parameters

Response status code 200: Successful operation

shelved-alarms-v2 parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| alarm-shelf-last-changed | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Last modification time of the alarm list, in format of 2019-03-30T08:21:46.000Z. This parameter is reserved. | - |
| shelved-alarm | ARRAY_REFERENCE | For details, see Table 1-46 |  | List of filtered alarms. | - |

Response status code 400: Bad request

For details, see the response message body.

Sample Response

Response status code 200: Successful operation

HTTP/1.1 200 OK 
Content-Type: application/json 
{ 
  "shelved-alarm": [{ 
    "resource-alarm-parameters": { 
      "perceived-severity": "major", 
      "is-cleared": false, 
      "status-change": [], 
      "last-changed": "2019-06-10T08:46:15.000Z" 
    }, 
    "x733-alarm-parameters": { 
      "event-type": "equipment-alarm" 
    }, 
    "operator-state-change": [], 
    "alarm-parameters": { 
      "ne-name": "7-241", 
      "location-info": "0-subrack-108-U1PSU-OTHER", 
      "native-probable-cause": "POWER_FAIL", 
      "probable-cause": "(1)The standby power is off;\n(2)The power board is failed;\n(3)The power supply of the board is aged.", 
      "root-cause-identifier": false, 
      "ems-time": "2019-06-10T13:59:06.853Z", 
      "alarm-serial-number": "39621", 
      "reason-id": 125, 
      "tenant-id": "", 
      "tenant": "", 
      "alarm-text": "Power failure" 
    }, 
    "common-alarm-parameters": { 
      "alt-resource": [], 
      "resource": "91a94f76-8b6f-11e9-853d-b77f2b1e7375", 
      "resource-url": "/restconf/v2/data/huawei-nce-resource-inventory:network-elements/network-element/91a94f76-8b6f-11e9-853d-b77f2b1e7375", 
      "related-alarm": [], 
      "alarm-type-qualifier": "268374017-125", 
      "impacted-resource": [], 
      "root-cause-resource": [], 
      "alarm-type-id": "equipment-alarm" 
    } 
  }] 
}

Response status code 400: Bad request

HTTP/1.1 400 Bad Request 
Date: Mon,30 Dec 2019 10:00:00 GMT 
Server: example-server 
Content-Type: application/json;charset=UTF-8 
Content-Length: 250 
Connection: keep-alive

Query Alarms based on Service Information

Typical Scenarios

The OSS needs to query all active alarms on a service (Client or OCh).

Functions

Query active alarms on a Client or OCh service based on the service UUID and type. The alarm model is consistent with IETF.

Constraints

1. Considering that one Client or OCh service usually has only a few alarms, this API does not support pagination.

2. This API can query alarms on only one service at a time.

HTTP Method

POST

URI

/restconf/v1/operations/huawei-nce-alarm:query-service-related-alarms

Request Parameters

body parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| service-alarms | Yes | REFERENCE | For details, see Table 1-32. |  | This API is used to query the active alarms on a service. | - |

service-alarms-input parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| ietf-alarms:input | No | REFERENCE | For details, see Table 1-33. |  | Request for querying service alarms. | - |

service-info parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| service-id | Yes | string | 36 characters. The value must meet the following regular expression: [0-9a-fA-F]{8}-[0-9a-fA-F]{4}-[0-9a-fA-F]{4}-[0-9a-fA-F]{4}-[0-9a-fA-F]{12}. | - | UUID of a Client or OCh service. | "3afff438-95c5-11ea-abe4-286ed4d4f408" |
| service-type | Yes | string | otn-trail | - | Service type. | "otn-trail" |

Sample Request

HTTP

POST /restconf/v1/operations/huawei-nce-alarm:query-service-related-alarms 
{ 
  "ietf-alarms:input": { 
    "service-id": "3afff438-95c5-11ea-abe4-286ed4d4f408", 
    "service-type": "otn-trail" 
  } 
}

Response Parameters

Response status code 200: Successful operation

service-alarms-output parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| ietf-alarms:output | REFERENCE | For details, see Table 1-16. |  | Response to the query of service alarms. | - |

Response status code 400: Bad request

For details, see the response message body.

Sample Response

Response status code 200: Successful operation

HTTP/1.1 200 OK 
Date: Mon,30 Dec 2019 10:00:00 GMT 
Server: example-server 
Content-Type: application/json;charset=UTF-8 
Content-Length: 250 
Connection: keep-alive 
{ 
	"ietf-alarms:output" : { 
		"alarm" : [ 
			{ 
				"time-created" : "2019-03-30T08:21:46.000Z", 
				"is-acked" : false, 
				"resource-alarm-parameters" : { 
					"perceived-severity" : "major", 
					"is-cleared" : false, 
					"status-change" : [ 
						{ 
							"perceived-severity" : "critical", 
							"alarm-text" : "Alarm cleared.", 
							"time" : "2019-03-30T08:21:46.000Z" 
						} 
					], 
					"last-changed" : "2019-03-30T08:21:46.000Z" 
				}, 
				"x733-alarm-parameters" : { 
					"event-type" : "equipment-alarm" 
				}, 
				"operator-state-change" : [ 
					{ 
						"operator" : "nbitest", 
						"state" : "ack", 
						"time" : "2019-03-30T08:21:46.000Z" 
					} 
				], 
				"alarm-parameters" : { 
					"ne-name" : "NE(9-95)", 
					"location-info" : "0-subrack-108-U1PSU-OTHER", 
					"native-probable-cause" : "POWER_FAIL", 
					"probable-cause" : "(1)The standby power is off;(2)The power board is failed;(3)The power supply of the board is aged.", 
					"root-cause-identifier" : false, 
					"ems-time" : "2019-03-30T08:21:46.000Z", 
					"alarm-serial-number" : "39621", 
					"reason-id" : 125, 
					"tenant-id" : "00000000-0000-0000-0000-000000000000", 
					"tenant" : "DefaultOrganization", 
					"alarm-text" : "Power failure", 
					"other-info" : "OrderNumber=ITSM-9FLOIBMX2PV1KU8C, Src=*.*.*.*, Dst=*.*.*.*, Index=31, Alarm Parameter II(hex) 0x00, ss=rrrrr", 
					"ip-address" : "127.0.0.1" 
				}, 
				"common-alarm-parameters" : { 
					"alt-resource" : [], 
					"resource" : "ad3028ba-c7d4-11ea-ad00-286ed489e617", 
					"resource-url" : "/restconf/v2/data/huawei-nce-resource-inventory:network-elements/network-element/ad3028ba-c7d4-11ea-ad00-286ed489e617", 
					"alarm-type-qualifier" : "100000-299", 
					"impacted-resource" : [ 
						 "NE(29-223)-NE(29-223)-OTS-1259" 
					], 
					"root-cause-resource" : [ 
						 "" 
					], 
					"alarm-type-id" : "security-service-or-mechanism-violation-alarm", 
					"layer" : "LR_Not_Applicable", 
					"md-name" : "Huawei/NCE", 
					"product-type" : "HUAWEI OSN902" 
				} 
			} 
		] 
	} 
}

Response status code 400: Bad request

HTTP/1.1 400 Bad Request 
Date: Mon,30 Dec 2019 10:00:00 GMT 
Server: example-server 
Content-Type: application/json;charset=UTF-8 
Content-Length: 250 
Connection: keep-alive

Update Alarm Information

Typical Scenarios

The OSS needs to update alarm information, such as additional information.

Functions

Update alarm information.

Constraints

1. This interface supports only a maximum of ten concurrent calls.

2. Only additional alarm information (such as ticket IDs) can be updated. Each call of this interface can update a maximum of 100 alarms at the same time.

HTTP Method

POST

URI

/restconf/v1/operations/huawei-nce-alarm:update-alarms

Request Parameters

body parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| update-alarm-input | Yes | REFERENCE | For details, see Table 1-36. |  | Update alarm information. | - |

update-alarm-input parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| input | Yes | REFERENCE | For details, see Table 1-37. |  | List of alarms to be updated. | - |

update-alarms parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| alarm | No | ARRAY_REFERENCE | For details, see Table 1-38. |  | Alarm to be updated. | - |

alarm parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| time-created | No | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Alarm generation time, that is, time when the alarm is generated for the first time or generated again after being cleared. The time format is 2019-03-30T08:21:46.000Z. If alarm status (is-cleared, perceived-severity, or alarm-text) changes, the time-created node will not be affected but the last-changed node will be updated. | "2019-03-30T08:21:46.000Z" |
| is-acked | No | boolean | - | - | Acknowledgment status of the alarm. | false |
| resource-alarm-parameters | No | REFERENCE | For details, see Table 1-39. |  | Alarm parameters in the resource view. | - |
| x733-alarm-parameters | No | REFERENCE | For details, see Table 1-41. |  | X.733 parameters of the alarm. | - |
| operator-state-change | No | ARRAY_REFERENCE | For details, see Table 1-42. |  | History of alarm operations performed by operators. | - |
| alarm-parameters | No | REFERENCE | For details, see Table 1-43. |  | Extended parameters in the northbound alarm model, compared with IETF. | - |
| common-alarm-parameters | No | REFERENCE | For details, see Table 1-44. |  | General alarm attributes. | - |

resource-alarm-parameters parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| perceived-severity | No | string | indeterminate minor warning major critical | - | Current alarm severity. | "major" |
| is-cleared | No | boolean | - | - | Clearance status. | false |
| status-change | No | ARRAY_REFERENCE | For details, see Table 1-40. |  | Alarm change history, including is-cleared, perceived-severity, and alarm-text changes. | - |
| last-changed | No | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Latest update time, in format of 2019-03-30T08:21:46.000Z. Change scenarios include is-cleared, perceived-severity, and alarm-text. For a fault alarm, enter its occurrence time. For a clear alarm, enter the clearance time. This parameter does not reflect the time of severity changes. | "2019-03-30T08:21:46.000Z" |

alarm-state-change-parameters parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| perceived-severity | No | string | indeterminate minor warning major critical | - | Alarm severity (changeable). | "critical" |
| alarm-text | No | string | 0 to 256 characters. | - | Alarm status change. This parameter takes effect only when the alarm experiences clearance, acknowledgment, or severity changes. | "Alarm cleared." |
| time | No | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Time when the alarm status changes, in the format of 2019-03-30T08:21:46.000Z. This parameter takes effect only when an event or alarm is reported, or an alarm experiences clearance, acknowledgment, or severity changes. | "2019-03-30T08:21:46.000Z" |

x733-alarm-parameters parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| probable-cause | No | int64 | - | - | Alarm cause. This parameter is reserved. For details about possible causes, see probable-cause in alarm-parameters. | - |
| event-type | No | string | other-alarm communications-alarm quality-of-service-alarm processing-error-alarm equipment-alarm environmental-alarm integrity-violation-alarm operational-violation-alarm physical-violation-alarm security-service-or-mechanism-violation-alarm time-domain-violation-alarm | - | Alarm type defined by ITU-T. | "equipment-alarm" |

operator-parameters parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| operator | Yes | string | 0 to 256 characters. | - | Alarm operator. | "nbitest" |
| text | No | string | - | - | Additional operation information. | - |
| state | Yes | string | none ack shelved unshelved closed | - | Alarm status from operators' perspective. The options are as follows: - none - ack - shelved - unshelved - closed | "ack" |
| time | Yes | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Operation time, in format of 2019-03-30T08:21:46.000Z. | "2019-03-30T08:21:46.000Z" |

alarm-parameters parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| repair-action | No | string | - | - | Alarm handling suggestions. | - |
| ne-name | No | string | 0 to 768 characters. | - | Name of the NE where the alarm is generated. If the alarm is reported from an NE, enter the NE name. If the alarm is reported from a system service, enter OSS. If the alarm is reported from the performance module, enter an NE name. | "NE(9-95)" |
| location-info | No | string | 0 to 2250 characters. | - | Alarm location information, which cannot be parsed and is in format of 8-N1EDQ41-5(SDH-5)-VC4:5-VC12:16[1-6-1]. | "0-subrack-108-U1PSU-OTHER" |
| native-probable-cause | No | string | 0 to 768 characters. | - | Alarm name, which briefly describes the related fault symptom. | "POWER_FAIL" |
| probable-cause | No | string | 0 to 65535 characters. | - | Possible causes of the alarm. | "(1)The standby power is off;(2)The power board is failed;(3)The power supply of the board is aged." |
| root-cause-identifier | No | boolean | - | - | Whether it is a root alarm. | false |
| ems-time | No | string | The value must meet the following regular expression: \\d{4}-\\d{2}-\\d{2}T\\d{2}:\\d{2}:\\d{2}(\\.\\d+)?(Z¦[\\+\\-]\\d{2}:\\d{2}). | - | Time when the alarm arrives at the EMS, in format of 2019-03-30T08:21:46.000Z. | "2019-03-30T08:21:46.000Z" |
| alarm-serial-number | Yes | string | 0 to 64 characters. | - | Alarm log SN. | "39621" |
| reason-id | No | int64 | - | - | Alarm cause ID. (One alarm may have multiple causes. This parameter is used to identify the specific cause of the alarm.) | 125 |
| tenant-id | No | string | 0 to 1536 characters. | - | Tenant ID, which is involved only in the tenant scenario. | "00000000-0000-0000-0000-000000000000" |
| tenant | No | string | 0 to 1536 characters. | - | Tenant name, which is involved only in the tenant scenario. | "DefaultOrganization" |
| alarm-text | No | string | 0 to 65535 characters. | - | Alarm description, which should describe the fault symptom and provide handling suggestions. | "Power failure" |
| other-info | Yes | string | 0 to 750 characters. | - | Alarm additional information | "OrderNumber=ITSM-9FLOIBMX2PV1KU8C, Src=*.*.*.*, Dst=*.*.*.*, Index=31, Alarm Parameter II(hex) 0x00, ss=rrrrr" |
| ip-address | No | string | - | - | IP address of the alarm source | "127.0.0.1" |

common-alarm-parameters parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| alt-resource | No | array<string> | Each item has 0 to 256 characters. | - | List of other identification modes for the resource object where the alarm is generated. | [] |
| resource | No | string | The value must meet the following regular expression: \\w{8}(-\\w{4}){3}-\\w{12}¦OSS. | - | Resource object of the alarm. | "ad3028ba-c7d4-11ea-ad00-286ed489e617" |
| resource-url | No | string | 0 to 4096 characters. | - | URL of the object associated with the alarm. | "/restconf/v2/data/huawei-nce-resource-inventory:network-elements/network-element/ad3028ba-c7d4-11ea-ad00-286ed489e617" |
| related-alarm | No | ARRAY_REFERENCE | For details, see Table 1-45. |  | Associated alarm. | - |
| alarm-type-qualifier | No | string | 0 to 256 characters. | - | Alarm type UUID. | "100000-299" |
| impacted-resource | No | array<string> | Each item has 0 to 1536 characters. | - | List of the resource objects that may be affected by the alarm. | - |
| root-cause-resource | No | array<string> | Each item has 0 to 256 characters. | - | List of resource objects for the root alarm. | - |
| alarm-type-id | No | string | other-alarm communications-alarm quality-of-service-alarm processing-error-alarm equipment-alarm environmental-alarm integrity-violation-alarm operational-violation-alarm physical-violation-alarm security-service-or-mechanism-violation-alarm time-domain-violation-alarm | - | Description of the alarm type. | "security-service-or-mechanism-violation-alarm" |
| layer | No | string | 0 to 256 characters. | - | Service layer. | "LR_Not_Applicable" |
| md-name | No | string | 0 to 256 characters. | - | Name of the system that sends the alarm. | "Huawei/NCE" |
| product-type | No | string | 0 to 192 characters. | - | NE type. | "HUAWEI OSN902" |

related-alarm parameters

| Parameter | Mandatory | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- | --- |
| root-alarm-serial-number | No | string | - | - | SN of the root alarm. | - |
| alarm-type-qualifier | No | string | - | - | Type of the associated alarm. | - |
| resource | No | string | - | - | Resource identifier of the associated alarm. | - |
| alarm-type-id | No | string | - | - | ID of the associated alarm. | - |

Sample Request

HTTP

POST /restconf/v1/operations/huawei-nce-alarm:update-alarms 
Content-Type: application/json 
{ 
  "input": { 
    "alarm": [ 
      { 
        "alarm-parameters": { 
          "alarm-serial-number": "1592055", 
          "other-info": "OrderNumber=ITSM-9FLOIBMX2PV1KU8C, Src=*.*.*.*, Dst=*.*.*.*, Index=31, \\nAlarm Parameter II(hex) 0x00, ss=rrrrr" 
        } 
      } 
    ] 
  } 
}

Response Parameters

Response status code 200: Successful operation

Response status code 207: Partially successful

Response status code 400: Internal Error

For details, see the response message body.

Sample Response

Response status code 200: Successful operation

HTTP/1.1 200 OK

Response status code 207: Partially successful

HTTP/1.1 207 Multi-Status 
Date: Mon,30 Dec 2019 10:00:00 GMT 
Server: example-server 
Content-Type: application/json;charset=UTF-8 
Content-Length: 250 
Connection: keep-alive

Response status code 400: Internal Error

HTTP/1.1 400 Bad Request 
Date: Mon,30 Dec 2019 10:00:00 GMT 
Server: example-server 
Content-Type: application/json;charset=UTF-8 
Content-Length: 250 
Connection: keep-alive

shelved-alarm parameters

| Parameter | Type | Value Range | Default Value | Description | Example |
| --- | --- | --- | --- | --- | --- |
| resource-alarm-parameters | REFERENCE | For details, see Table 1-18. |  | Alarm parameters in the resource view. | - |
| x733-alarm-parameters | REFERENCE | For details, see Table 1-20. |  | X.733 parameters of the alarm. | - |
| shelf-name | string | - | - | List of excluded alarms. | - |
| operator-state-change | ARRAY_REFERENCE | For details, see Table 1-21. |  | History of alarm operations performed by operators. This parameter is reserved. | - |
| alarm-parameters | REFERENCE | For details, see Table 1-22. |  | Extended parameters in the northbound alarm model, compared with IETF. | - |
| common-alarm-parameters | REFERENCE | For details, see Table 1-23. |  | General alarm attributes. | - |